Submission form for phytosanitary treatments

Vapour heat (hot steam) treatment of coniferous bark for the elimination of Bursaphelenchus xylophilus (pine wood nematode – PWN)

(Reviewed by TPPT March 2016)

Name of Country/RPPO: PORTUGAL/EPPO______________________________________________

Click here to find the IPPC Procedure Manual for Standard Setting on the IPP (www.ippc.int), where you can download this form.

Submission number (Secretariat Use Only):

Complete the following form, preferably in electronic format, and submit by e-mail to the IPPC Secretariat (ippc@fao.org).

Please use one form per phytosanitary treatment. An electronic version of this form is available on the International Phytosanitary Portal (IPP) at https://www.ippc.int/en/publications/1089/. Incomplete submissions will be returned. Please save the completed submission form with the following file name: COUNTRY or RPPO NAME –Title of treatment.doc, prior to submitting to the IPPC Secretariat via e-mail.

(Text in brackets given for explanatory purposes)

| Name of treatment | Vapour heat (hot steam) treatment of coniferous bark for the elimination of Bursaphelenchus xylophilus (pine wood nematode – PWN) |
| --- | --- |

Submitted by: Direção Geral de Alimentação e Veterinária (DGAV)

Contact: (Contact information of an individual able to clarify issues relating to this submission, including sources of efficacy data) Name: Clara Serra Position and organization: Senior plant health inspector and coordinator of treatment strategy for PWN control Mailing address: DGAV, Tapada da Ajuda, edifício 1, 1349-018 Lisboa, Portugal Phone: +351 21 361 32 79 Fax: E-mail: cserra@dgav.pt

# Treatment description

| Active ingredient | N/A |
| --- | --- |
| Treatment type | Physical (vapour heat) |
| Target pest | Bursaphelenchus xylophilus |
| Target regulated articles | Coniferous bark |
| Treatment schedule | Exposure to a continuous system, where bark is heated in an injection chamber by hot steam at a minimum temperature of 100ºC, followed by a treatment of at least 30 consecutive minutes above 64ºC. |
| Other relevant information | The elimination of PWN is considered effective in the framework of ISPM-15 for Wood packaging material if temperature and time complies with 56ºC/30min across the entire commodity |
| References | - Portuguese standard NP 4515/2020 - Softwood bark and wood chips Phytosanitary Heat Treatment for the elimination of the pinewood nematode (Bursaphelenchus xylophilus) and other harmful organisms. (Unofficial English version). - Fonseca L et al., Coniferous bark hot steam treatment for the elimination of the pinewood nematode. In: Schröder, T. (ed.), Pine Wilt Disease Conference 2013, pp. 44-45, Braunschweig. - List of other supporting documents mentioned in the text |

The following form must be completed in accordance with ISPM 28 Phytosanitary treatments for regulated pests, the IPPC Strategic Framework and the Procedure and criteria for identifying topics for inclusion in the IPPC standard setting work programme.

Copies of all relevant supporting information and publications should be supplied with the treatment submission, preferably in PDF format, for ease of subsequent distribution.

The following form refers to the relevant sections of ISPM 28 and are numbered accordingly.

3.2 Efficacy data in support of the submission of a phytosanitary treatment

The source of all efficacy data (published or unpublished) should be provided in the submission. Supporting data should be presented clearly and systematically.

3.2.1 Efficacy data under laboratory/controlled conditions (Treatments may be considered without efficacy data under laboratory/controlled conditions if sufficient efficacy data is available from the operational application of the treatment (section 3.2.2) and if no data under laboratory/controlled conditions exists this section may be left blank.)

Pest information

Identity of the pest to the appropriate level, life stage, and if a laboratory or field strain was used

Conditions under which the pests are cultured, reared or grown

Biological traits of the pest relevant to the treatment

Method of natural or artificial infestation

Determination of most resistant species/life stage (in the regulated article where appropriate)

Regulated article information

Type of regulated article and intended use

Botanical name for plant or plant product (where applicable)

Conditions of the plant or plant product

Experimental parameters

Level of confidence of laboratory tests provided by the method of statistical analysis and the data supporting that calculation

Experimental facilities and equipment

Experimental design

Experimental conditions

Monitoring of critical parameters

Methodology to measure the effectiveness of the treatment

Determination of efficacy over a range of critical parameters, where appropriate

Methodology to measure phytotoxicity, when appropriate

Dosimetry system, calibration and accuracy of measurements,

3.2.2 Efficacy data using operational conditions (historical data, may in some cases substitute for the requested information below)

Pest information

Identity of the pest to the appropriate level, life stage, and if a laboratory or field strain was used

Bursaphelenchus xylophilus, Pine Wood Nematode, all stages are eliminated. The proposed combination of time and temperature in this treatment is above the internationally accepted combination of 56ºc during 30 consecutive minutes, across the entire commodity.

Conditions under which the pests are cultured, reared or grown

N/A

Biological traits of the pest relevant to the treatment

N/A

Method of natural or artificial infestation

Samples infected with PWN were prepared with naturally infected wood pieces and additional individuals (inoculated from fungus culture) of different life stages, including JIII stage as this stage is considered the most resistant. This methodology is the one used to prepare the infected samples used in the assays referred in the experimental design below.

Determination of most resistant species/life stage (in the regulated article where appropriate)

As referred in scientific literature, JIII juvenile stage is the most resistant one.

Regulated article information

Type of regulated article and intended use

Coniferous bark intended use for growing substrate or growing medium for plants or mulching purposes.

Botanical name for plant or plant product (where applicable)

Pinopsida class

Conditions of the plant or plant product

Material resulting from debarking activity of Pinopsida logs in sawmills.

Experimental parameters

Level of confidence of laboratory tests provided by the method of statistical analysis and the data supporting that calculation

As referred in the experimental design below, a preliminary test and several biological assays were performed to demonstrate the efficacy of hot saturated steam on coniferous bark to eliminate PWN. Probit 9 efficacy was demonstrated (100% mortality of more than 100 000 test organisms).

Experimental facilities and equipment

Facilities with a dedicated area to receive the untreated bark and to perform its screening. Before proceeding to the heat treatment system, the bark is cleaned to remove stones, branches, metallic objects, plastics and other large contaminants, sand, fine dust of bark, wood particles (and others, not compatible with the final product). After screening, calibration is also applied to separate the bark into different calibres (as settled in point 9 of NP 4515/2020). Dedicated storage area for the treated bark, complying with specific rules to avoid any contamination by PWN. The time of storage is also limited, and transport must be in closed containers (as settled in point 11 of NP 4515/2020). Hygiene measures to avoid recontaminations are described in point 12 of NP 4515/2020. The equipment is composed by: - heat source- burning boiler for steam production - heating and treatment chambers with steam injectors; - temperature measurement sensors; - automatic temperature recording system; - Automatic treated volumes recording system (m3). More detailed description is settled at point 4 and 5 of the NP 4515/2020. All approved industrial units must comply with the above facilities and equipment requirements.

Experimental design

In order to evaluate the possible efficacy of hot saturated steam on coniferous bark to eliminate PWN, two main factors have to be fulfilled. The temperature and exposition time by one side, and the mortality achieved by the other, are crucial. To achieve a successful result, which is the mortality of the pest, thermal and biological validation must be ensured. For this purpose, a first assay was performed in experimental conditions. A preliminary test was conducted in an experimental chamber where the heat source was an electric steam generator. In this test, samples of bark and a mixture of infected bark and wood chip were placed separately in two net bags on plastic trays. The chamber was closed with these materials only and programmed to reach an internal temperature of 85°C and to switch off 30 minutes after this temperature had been reached. The heat supply was provided by injecting saturated steam at 100°C. The temperature of the material being tested was recorded using two automatic ‘data-logger’ button sensors/recorders set to record every minute, which were placed on the samples of bark and bark mixed with wood chip. Further details on this experiment can be found in doc. 1. The results showed that the bark and wood chip remained at temperatures above 65°C for a little over 30 minutes, and above 56°C for around 1 hour, meeting the regulatory conditions for bark and wood chip treatment in terms of PWN presence. Nematological analyses of both samples (bark and bark + wood chip) were performed in three phases: 1 – before treatment; 2 – after steam treatment; and 3 – after an incubation period of 15 days at a temperature of around 25°C. Nematodes were extracted from both samples using the tray method (Whitehead & Hemming, 1965) over 48 hours, and nematodes detected were identified and counted. Before treatment, nematodes of the Rhabditidae and Aphelenchoididae families and others of the Bursaphelenchus genus were detected in the pine bark sample (8.70 kg) (1785 nematodes/100g, corresponding to approximately 149 940 nematodes) and nematodes from the Rhabditidae and Aphelenchoididae families including B. xylophilus and other species of the Bursaphelenchus genus were detected in the bark and wood chip sample (1230 B. xylophilus/100g and 735 other nematodes/100g, corresponding to approximately 111 720 nematodes). Nematodes were not detected after heat treatment without incubation and after incubation. Results are presented in table 1 of doc. 1. Probit 9 efficacy was demonstrated (100% mortality of more than 100 000 test organisms). As these preliminary results were very promising in what concerns the efficacy of the treatment, additional trials were performed, these already under operational conditions, with industrial equipment. The equipment is described under “Experimental facilities and equipment” item above. Trials at industrial level: Trials with industrial equipment: a first assay was conducted with 6 bags containing wood chips and bark (800g/bag) infested with 118 000 B. xylophilus (80% of those being JIII stage juveniles which is the most resistant stage) and 5 000 other nematodes, including other Bursaphelenchus species. The bags were submitted to a temperature above 92ºC during around 50 minutes, monitored by 5 fixed sensors, as shown in graphic of page 38 of doc. 2. Nematological analyses of all the samples were performed before and after the treatment: all the samples were analysed before treatment and, after treatment, 3 samples were analysed immediately, and the other 3 samples were analysed after an incubation period of 15 days at a temperature of around 25°C. The results are presented in table of page 39 of. Doc. 2, where it can be seen that nematodes were present before treatment, above 100 000 individuals of PWN, and after treatment, with or without incubation, no nematodes were detected in any sample. Probit 9 efficacy was demonstrated (100% mortality of more than 100 000 test organisms). A second assay was conducted with 6 bags containing wood chips and bark (700g/bag) infested with 131 000 B. xylophilus (more than 80% of those being JIII stage juveniles which is the most resistant stage) and 3 600 other nematodes, including other Bursaphelenchus species. In this trial a portable sensor was placed inside each bag, to register the temperature reached in each bag during the period of the treatment. The bags were submitted to a temperature above 92ºC during around 50 minutes, monitored by 5 fixed sensors, as shown in graphic of page 44 of doc. 2. Concerning the thermal profile of the 6 portable sensors, that were recovered after treatment, we can conclude that all the sensors registered a temperature above 86,5ºC during 30 consecutive minutes, as it can be seen in graphics in pages 42 and 43 of doc. 2. Nematological analyses of all the samples were performed before and after the treatment: all the samples were analysed before treatment and, after treatment, 3 samples were analysed immediately, and the other 3 samples were analysed after an incubation period of 15 days at a temperature of around 25°C. The results are presented in table of page 45 of. Doc. 2, where it can be seen that nematodes were present before treatment, above 100 000 individuals of PWN, and after treatment, with or without incubation, no nematodes were detected in any sample. Probit 9 efficacy was demonstrated (100% mortality of more than 100 000 test organisms). A third assay was conducted with 2 bags containing a portable sensor each, one with a wood piece where the sensor was inserted inside, and another with a bark piece, also with a sensor inserted inside the piece. This experiment was settled to evaluate the temperature in the core of the piece during treatment. The thermal profile of the 2 portable sensors, that were recovered after treatment, shows that both sensors registered a temperature above 75,6ºC during 30 consecutive minutes, as it can be seen in graphic in page 47 of doc. 2. As an overall conclusion, in all the situations the temperature reached during treatment using vapour heat is above the minimum temperature internationally recognised to kill the PWN, and the biological assays demonstrate a 100% efficacy to eliminate Bursaphelenchus xylophilus, across the entire treated volume. In this way, these results were presented and accepted by the European Commission as valid for coniferous bark treatment and its circulation in the entire EU. DGAV produced written technical requirements for bark treatment in Portuguese companies, for their registration and to perform the bark treatment according to those requirements. Later, a national standard for the phytosanitary heat treatment of bark was elaborated, in collaboration with several national entities, including DGAV, the academia and the registered companies. The Standard is NP 4515/2020, and a courtesy translation document is provided in annex. This standard is aligned with the technical requirements established by DGAV and provides guidance to the other Member States of the EU. Currently, we have 6 companies registered for bark treatment and the equipment of each facility is evaluated at an approval stage and, at least, annually to verify the compliance with the NP 4515/2020, performed by an accredited laboratory. This evaluation includes the thermal validation and the validation of the recording system of treatment capacity (volume of treated bark per time unit). The evaluation is done by a laboratory recognised by DGAV, that produces the reports. The English versions of the six reports issued by that laboratory for each one of the six approved facilities are in annex, doc. 3 to 8. A biological evaluation is also performed to validate the efficacy of the treatment on elimination of PWN at the approval stage of the facility. This evaluation is done by another laboratory recognised by DGAV, that produces a report for each facility. For this biological evaluation performed in each of the companies, 15 samples are prepared, containing each one a portable temperature sensor to demonstrate that both temperature and time exceeds the 56ºC/30min across the entire commodity. This methodology and the efficacy data are shown in the reports, doc. 9 to 13.

Experimental conditions

The equipment that provides the heat treatment of the bark in a continuous way has 5 fixed temperature sensors: 2 in the heating (injection) chamber and 3 in the treatment chamber, located along the chamber. However some equipments have a double heating chamber capacity, with 4 sensors in the heating chamber and 3 in the treatment chamber.

Monitoring of critical parameters

Both temperature in the chambers and volumes of bark treated in a given period of time (m3/hour) are measured, automatically recorded and data stored by the equipment. At an industrial stage, records include the identification of the business company (operator code), time (day, hour, minute), serial number of the sensor and respective temperature data. Maximum interval between temperature value recordings during treatment is 10 minutes. Data on the effective accumulated volume of treated bark is also stored. All data can be exported by the system for reading and analysis. These requirements are also mentioned in the NP 4515/2020, point 7 and 8.

Methodology to measure the effectiveness of the treatment

Readings of the automatically recorded temperature data must show that values are equal to or higher than 64ºC for a continuous period of at least 30 minutes along the treatment chamber (standard parameters). In addition, the amount of treated bark per hour is verified not to have exceeded the defined capacity of the equipment (steam boiler).

Determination of efficacy over a range of critical parameters, where appropriate

Thermal validation Each heat treatment system is submitted to a validation study by an accredited laboratory annually, which consists on the introduction of portable recording sensors (button type) in the process, for automatic temperature recording, while, mixed with bark or wood chips, traveling through all the system, since the initial heating phase until the end of the treatment, allowing detailed information of the temperature profile in function of the time to be recorded and analysed. In the continuous treatment system, the process validation is done with one sensor for each 4 cubic meter per hour (m3/h) of bark or wood chips, being, with this purpose, introduced in the bark or wood chips system one sensor every 1 minute. These sensors are collected at the end of the treatment, read in a computer and the relationship of temperatures with the road time in the system studied. In the thermal validation study of each treatment system, the portable sensors of the thermal study and the fixed sensors of the unit process control, both must be working simultaneously. The confront between the measurements taken by the two systems is considered and expressly stated in the validation report. The records of these sensors also demonstrate the duration of the treatment (travel time can be estimated between time when entering in the heating chamber and out of the treatment chamber. This is defined as the exposition time). Temperature of the portable sensors must demonstrate that the temperature and time exceeds the 56ºC/30min across the entire commodity. The maximum amount of bark to be treated is settled for each equipment; depending on the capacity of the steam boiler, the volume measured by the automatic system can never exceed that value. This evaluation shall be assessed by the referred laboratory. Each industrial facility is equipped with an automatic measurement and recording system for treated bark. Annually, the equipment capacity of the volume of treated bark needs to be verified. On a routine basis, for the efficacy of the treatment the following items are checked, and both have to be simultaneously in compliance: - the temperature recorded by the 3 fixed sensors (above 64ºC on the 3 sensors simultaneously) - volumes of bark treated (m3 per hour) registered by an automatic system– in order to be in conformity with the exposition time of at least 30 minutes above 64ºC each equipment has a different performance, so the adequate volume varies for each one. To check the conformity of the treatment, the records of volume treated per hour are compared with the standardised volume settled for each equipment according to its specifications. The volume treated per hour cannot exceed the standardised amount settled for that equipment, otherwise the treatment is not valid.

Methodology to measure phytotoxicity, when appropriate

N/A

Dosimetry system, calibration and accuracy of measurements

As requested in NP 4515/2020, the temperature sensors associated with the automatic control system are subjected to initial calibration and every 12 months maximum, by an accredited laboratory. The data acquisition regarding volume of treated bark shall be done with appropriate sensors whose information will be processed by computer for automatic recording of the effective accumulated volume. This evaluation shall be assessed by the referred laboratory.

Factors that affect the efficacy of the treatment

The industrial facilities have an adequate area to receive the untreated bark and to perform its screening. Before entering the heat treatment system, the bark is cleaned to remove stones, branches, metallic objects, plastics and other large contaminants, sand, fine dust of bark, wood particles (or others not compatible with the final product). After screening, a calibration system is also applied to separate the bark into different calibres (as stated in point 9 of NP 4515/2020) The industrial unit must have a storage area for the treated bark, submitted weekly to adequate hygiene measures, to avoid any new contamination by PWN. The time of storage is also limited, and transport and expedition must be done in closed containers submitted also to adequate hygiene measures (as stated in point 11and 12 of NP 4515/2020).

Special procedures that affect the success of the treatment, if applicable

Hygiene measures are fundamental to avoid recontaminations and so not to compromise the success of the treatment. In particular, the use of different loaders should be specifically allocated to treated area and non-treated area. Also, containers used for bark transportation should be previously subject to hygiene measures before loading. All the conveyor mechanisms must be cleaned weekly.

3.3 Feasibility and applicability (Information should be provided where appropriate on the following items)

Procedure for carrying out the phytosanitary treatment

Since the first detection of PWN in Portugal actions were directed to control and eradicate the nematode. In this framework, European legislation was published to determine emergency measures against PWN, whose actual regime is provided by Decision 2012/535/EU, in its updated version. In the Portuguese legislation, the Decree-law nº95/2011 was published to reinforce at national level the established measures to control the nematode and avoid its spread to other countries. In this framework, WPM and bark of coniferous species movement out of the demarcated area was conditioned to its prior phytosanitary treatment. In the case of bark and chips the rationale of the phytosanitary treatment was established by the Portuguese Standard NP 4515/2020, that describes with detail the method for applying the treatment of Coniferous bark in a commercial setting, as well as the equipment required; also, the conditions that the approved facilities must comply to are settled. Based on these requirements DGAV also published technical guidelines specific for the heat treatment of bark.

Cost of typical treatment facility and operational running costs if appropriate

The prices for the boiler and heating and treatment chambers are around 500 thousand euros. The energetic costs of treatment are around 3,5 euros/m3 (using biomass or pellets).

Commercial relevance, including affordability

The export of Coniferous bark from Portugal to other countries of the European Union can only be done if it is heat treated in accordance with the NP 4515/2020, based on the official technical requirements, that were recognised by the European Commission. Some other countries outside the European Union accept the Portuguese bark treated, being the treatment officially validated by the issuance of a Phytosanitary certificate. However, due to the fact that there is no internationally recognised standard, several countries refuse to accept the Portuguese treated bark. In what concerns affordability, there are actually 6 companies approved in Portugal and for them there is an economic relevance for the selling of this product, considering that there is a market, which is highly specialised, for the supplying of a specific product, namely for orchids production, which is a valuable commodity.

Extent to which other NPPOs have approved the treatment as a phytosanitary measure

All the 26 countries that are Member States of the European Union and also Japan, Taiwan, USA, UAE, UK, China, Israel, Lebanon, Dubai and Qatar accept the Portuguese treated bark.

Availability of expertise needed to apply the phytosanitary treatment

If the applicable rules and the technical requirements established by DGAV are followed by the operators, no particular expertise is required. At least on an annual basis there are supervision actions taken by the NPPO (DGAV) on the commercial companies operating the bark heat treatment.

Versatility of the phytosanitary treatment

This equipment allows different combinations of temperature and time (if above 64ºC/30 minutes) in case the importing country requests so. Wood chips may also be subject to this treatment effectively, with the establishment of specific requirements.

The degree to which the phytosanitary treatment complements other phytosanitary measures

Even though subject to validation, this treatment has an action against other pests that may occur on bark, mainly insect of all stages, due to the temperature and time exposition. Additional studies might be required to ensure efficacy.

Summary of available information of potential undesirable side-effects

N/A

Applicability of treatment with respect to specific regulated article/pest combinations

Only the bark /PWN combination was evaluated.

Technical viability

The equipment is available at commercial level, can be acquired internationally, and used with the supply of electric power.

Phytotoxicity and other effects on the quality of regulated articles, when appropriate

N/A

Consideration of the risk of the target organism having or developing resistance to the treatment

N/A

# Send submissions to:

E-mail: ippc@fao.org		Mail: IPPC Secretariat (AGPP)

(preferred)		Food and Agriculture Organization of the UN

Viale delle Terme di Caracalla, 					00153 Rome, Italy