| [1] | ISPM 15: 2009 draft Annex: Process for testing new treatments for ISPM 15 (2006-010) |
| --- | --- |
| [2] | Publication history |
| [3] | Date of this document; 2016-06-09. Document category; Draft new Annex to ISPM 15:2009. Current document stage; Review by TPFQ at June 2016 meeting in Canada. Major stages; 2006-04 CPM-1 (2006) added: Revision of ISPM 15 (Regulation of wood packaging material in international trade): Criteria for treatments for wood packaging material in international trade (2006-010) 2008-12 TPFQ changed title to: Treatment criteria for ISPM No. 15 2009-05 SC reviewed draft of Treatment criteria for ISPM No. 15 2009-07 TPFQ changed title to: Submission of new treatments for inclusion in ISPM No 15 2010-05 SC approved for MC 2010-06 MC 2011-05 SC-7 reviewed 2011-07 to SC via e-decision 2011_eSC_Nov_02 2013-06 TPFQ reviewed IFQRG reports and revised draft ISPM. Title changed to: Process for developing new treatments for ISPM 15 2014-06 TPFQ reviewed text and revised draft ISPM 2016-06 TPFQ reviewed text and revised draft ISPM. Notes; SC via e-decision requested IFQRG to discuss the technical issues identified by the SC-7 and provide their recommendations to the appropriate technical panel (TPFQ and/or) TPPT through the Secretariat. 2011-09 IFQRG reviewed- Brisbane list of target pest groups developed 2012-09 IFQRG reviewed- Concept of Cardiff Protocol discussed 2012-12 TPPT agreed that the concept of the Cardiff Protocol would help develop more appropriate treatment efficacy requirements for target pests. References; Haack R.A., Uzunovic A., Hoover K., Cook J.A. (2011) Seeking alternatives to probit 9 when developing treatments for wood packaging materials under ISPM No. 15. Bulletin OEPP/EPPO Bulletin 41: 39–45. Schortemeyer M., Thomas K., Haack R. A., Uzunovic A., Hoover K., Simpson J. A., Grgurinovic C. A. (2011) Appropriateness of probit-9 in the development of quarantine treatments for timber and timber commodities. Journal of Economic Ecology 104: 717-731. Ormsby (2022) Elucidating the efficacy of phytosanitary measures for invasive alien species moving in wood packaging material. Journal of Plant Diseases and Protection. https://doi.org/10.1007/s41348-022-00571-1. |
| [4] | Introduction |
| [5] | When developing new phytosanitary treatments for ISPM 15, treatment developers should provide confidence that the pest risks of a wide variety of wood packaging material from all regions of the world have been addressed. |
| [6] | New phytosanitary treatments for inclusion in ISPM 15 are submitted by NPPOs or RPPOs for evaluation against the requirements outlined in ISPM 28. In addition procedures used to evaluate treatment submissions are available in the IPPC Procedure Manual Standard Setting. The stepwise guidance in this document on the treatment testing process is provided for NPPOs or RPPOs, treatment developers and researchers to harmonise work on the development of phytosanitary treatments to be submitted. |
| [7] | Treatment developers may need to collaborate nationally or internationally with scientists, technical professionals or NPPOs to gather data required to support the international adoption of a treatment. Treatment developers are encouraged to consult with experts (e.g. statisticians and pest biologists) at an early stage in the process in order to design experiments. During the treatment testing process, treatment developers may also need to collaborate with international experts to undertake supporting studies that may not be easily accomplished locally (e.g. specific pests for testing may not be available). If additional clarification on the process of testing, submission and evaluation of phytosanitary treatments is required, treatment developers should consult their NPPO, who may seek additional information from the IPPC Secretariat. |
| [8] | The ISPM 15 treatment testing process relies on the principle that all sources of existing relevant information should be considered to support each step in the process. Research may be required where existing information is insufficient. |
| [9] | The following elements that could affect treatment efficacy should be addressed in the development of an ISPM 15 phytosanitary treatment: |
| [10] | - pests likely to be associated with wood packaging material used in international trade; |
| [11] | - pest life stages most likely to be associated with wood packaging material used in international trade; |
| [12] | - wood species and characteristics (e.g. hardwood vs. softwood, sawn wood vs. round wood used as dunnage, moisture content, presence/absence of bark); |
| [13] | - environmental conditions (e.g. temperature, humidity, atmosphere). |
| [14] | Overview of the Process for Treatment Testing |
| [15] | Pests that should be used in the treatment testing process are presented in Table 1. These pests are associated with wood used for wood packaging material and affect forest trees. |
| [16] | Table 1. Organisms to be used in the treatment testing process. |
| [17] | Type of organism; Species, genera or family to be used. Insects; Bostrychidae, Buprestidae, Cerambycidae, Curculionidae and Siricidae.. Fungi; Heterobasidion sp., Ceratocystis sp.. Nematode; Bursaphelenchus xylophilus. |
| [18] | The treatment testing process is divided into four steps: |
| [19] | Step 1. Screening - Laboratory screening intended to identify the pest and its life stage (associated with wood packaging material) most tolerant to the phytosanitary treatment tested and the effective level of that treatment (i.e. dose, concentration, rate of application, duration, etc. that results in complete mortality of that pest at that life stage); |
| [20] | Step 2. Effect of physical parameters - Identification of the performance of the phytosanitary treatment under differing physical parameters of the environment in which the treatment is applied; |
| [21] | Step 3. Validation of the effective treatment level - Validation of the effective treatment level found in step 1 and 2 which results in complete mortality[1] under laboratory conditions; |
| [22] | Step 4. Validation under operational conditions - Testing under operational conditions to confirm the efficacy of the treatment during its use in the production of wood packaging material. |
| [23] | During the treatment testing process, controls should be used as appropriate for each replicate. Treatment developers should fully document the activities conducted under each step. Clear justification for deviations from the requirements (e.g. exclusions from the list of pests described in Table 1 or the use of substitute species) should be provided. |
| [24] | [1] footnote - Throughout this annex the term “mortality” is used to describe successful outcome of treatments. It should be noted however that successful phytosanitary treatments may not necessarily result in killing pests, but may inactivate, remove, devitalize or render them infertile (ISPM 5). However for the purposes of phytosanitary treatments in ISPM 15, mortality should be considered the successful outcome. |
| [25] | [include in footnote] Similarly, the term “survivors”, which is used in this annex to describe an unsuccessful outcome of treatment should not be understood as covering only the individuals that have not been killed by the treatment, but also those that have not been inactivated, removed, devitalized, or rendered infertile in cases where inactivation, removal, devitalization, or rendering infertile all pests is considered a treatment success. As the expected outcome of treatments for wood packaging material is the killing of all targeted pests, presence of survivors would usually indicate that the treatment has failed or has not been applied. |
| [26] | When assessing the treatment results, only a complete mortality of pests with no survivors should be considered a success of a replicate. In some cases, for nematodes or fungi, the evaluation of treatment results performed immediately after treatment may misrepresent the treatment success or failure. Some apparent survivors may die subsequently and, conversely, those that may appear moribund may recover. For that reason mortality of pests may need to be confirmed by the failure of recovery of pests determined by using appropriate testing methods. It should be clearly demonstrated in the treatment submission that treatment success occurs consistently throughout the entire profile of the wood. |
| [27] |  |
| [28] | Step 1: Screening |
| [29] | The screening is intended to gather data from the literature or under laboratory conditions to identify the effective treatment level for the most tolerant pest at the most tolerant life-stage. That life stage will be used for testing in Steps 2, 3 and 4. Screening should follow the data gathering requirements prescribed in ISPM 28. Evidence on treatment efficacy should be provided on at least one species from each pest group listed in Table 1. Reasons for the selection of the species of test organisms should be explained. Available data on the biology of the organism and resistance or tolerance to specific treatments should be used to guide or support the selection of the test organisms. |
| [30] | To identify the effective treatment level for the most resistant life stage of a pest likely to be associated with wood packaging material in international trade, data (from existing literature or laboratory research) should be provided that demonstrates all life stages of the eight selected pests have been killed at a specific treatment level when exposed to a range of treatment levels. It should be noted that one treatment level exceeding the level at which complete mortality was achieved, should be included in the testing to further confirm the level at which complete mortality is achieved. However, if information indicates that certain life stages are not relevant to the proposed treatment, these may be excluded, provided technical justification is included with the treatment submission. |
| [31] | For each variable in the test (e.g. dose, concentration, life stage, etc.) a minimum of five test units should be used. For insects, the test unit should be the single individual. For fungi or nematodes, the test unit should be a colonized piece of wood as it is not practical to handle these organisms individually. Individuals or isolates used for testing should have the quality, vigour and stability appropriate to naturally occurring organisms. Appropriate hosts should be used for each pest species to ensure an appropriate pest response is achieved. Some organisms, for example fungi and nematodes, should be tested only in vivo (in wood) unless evidence is provided that in vitro testing (e.g. fungi grown on nutrient agars in Petri plates) provides equivalent and acceptable results. Reasons for the selection of isolates should be clearly documented in the submission. |
| [32] | The results of the screening should document the treatment responses among the eight pests selected for testing. The screening should also indicate which of the pest life stages tested is the most tolerant (i.e. the pest life stage requiring the highest treatment level to achieve treatment success). |
| [33] | Step 2: Effect of physical parameters |
| [34] | Step 2 identifies the minimum treatment level which must be delivered throughout the profile of the wood tested under differing physical parameters. Treatment efficacy may be limited by a number of factors such as: |
| [35] | - a fumigants ability to penetrate wood; |
| [36] | - a fumigant may be diluted by substances in the wood; |
| [37] | - a treatment may be incapable of killing pests at a particular temperature or moisture content. |
| [38] | Treatment developers should therefore conduct testing of the treatment efficacy under differing physical parameters which may include: |
| [39] | - Temperature of the wood and the ambient air; - Composition of the atmosphere (e.g. levels of oxygen or carbon dioxide, vacuums, etc.); |
| [40] | - Moisture inside and at the surface of the wood (e.g. wood moisture contents of freshly cut and dried wood); |
| [41] | - Dimensions of wood (e.g. sizes of wood used for pallets, boxes or dunnage); |
| [42] | - Anatomy and density of wood (e.g. ring porous, diffuse porous, sapwood, heart-wood, etc.); |
| [43] | - Wood species (e.g. tropical and temperate species; coniferous and broadleaf species); |
| [44] | - Presence or absence of bark. |
| [45] | In step 2 a single piece of wood used for testing should be considered a test unit. At least 5 test units should be used for testing a single level of a physical parameter (e.g. one level of temperature, one species of wood etc.). |
| [46] | It should be demonstrated in the submission that the treatment is capable of killing pests throughout the profile of the wood considering various physical parameters. This may be achieved by monitoring pest mortality or demonstrating that the effective treatment level can be delivered throughout the entire profile of the wood. The effective treatment level determined in Step 2 may differ from the effective treatment level determined in Step 1 (usually it may be higher), as a result of the physical parameters. |
| [47] | If available information indicates that certain physical parameters are not relevant and were excluded from testing, the circumstances under which these do not affect the treatment should be described in the treatment submission. |
| [48] | Step 3 – Validation of the effective treatment level |
| [49] | Step 3 validates the effective treatment level identified in step 1 and 2 for the most tolerant pest life stage. A piece of wood infested by pests should be considered a test unit. The size of the pieces of wood used for testing should correspond to the size of wood packaging material used in international trade. Testing is achieved by exposing a minimum of 60 test units to the effective treatment level under laboratory conditions. An effective treatment level may be considered validated under laboratory conditions when there are no survivors in any of the test units. This step may not be required if the treatment developer is confident they have identified the effective treatment level. |
| [49] | Step 4 – Validation under operational conditions |
| [50] | Step 4 validates the effective treatment level under operational conditions, by exposing the most tolerant pest life stage in units of wood (of a size and nature normally used for wood packaging material) to the effective treatment level. This should be done under a range of conditions which may be found in practice in treatment facilities, including conditions most likely to result in the treatment being unsuccessful. Treatment developers should test pieces of wood infested with the most tolerant life stage of the pest at population or infestation levels normally expected in untreated wood. Parameters used for testing may determine the conditions for the use of the treatment once approved (e.g. minimum wood temperature at treatment of 20°C or greater). |
| [51] | In Step 4 a single piece of wood is considered a test unit. The number of test units (pieces of wood) to be used for testing should be determined taking into account the actual average level of infestation by insects found in the pieces of wood prepared for testing. For example if the infestation level of pieces of wood prepared for testing by a pest is found to be 4 per test unit and the required number of individuals of the pest to be used in the testing is 1,200, then the number of test units of wood exposed to the treatment in Step 4 should be greater than 300. Test units should be split into at least three replicates with a control per replicate e.g. 100 per replicate from the above example of 300 test units. These replicates should be exposed to the treatment separately. |
| [52] | If the most tolerant pest identified in Step 1 is Heterobasidion sp., Ceratocystis sp. or Bursaphelenchus xylophilus, a total of 180 test units of infested wood should be treated in at least three replicates (e.g. 60 test units per replicate) with appropriate controls per replicate. Each of these replicates should be exposed to the treatment separately. |
| [53] | If the most tolerant pest identified in Step 1 is an insect, the minimum number of individuals for testing is specified in Table 2. |
| [54] |  |
| [55] |  |
| [56] |  |
| [57] |  |
| [58] |  |
| [59] |  |
| [60] | Table 2. Minimum number of individuals required for testing the insect families provided in table 1, assuming no treatment failures (e.g. survivors) occur. Insect families; Minimum number of individuals. Bostrychidae; 6,188. Buprestidae; 5,700. Cerambycidae; 7,470. Curculionidae; 997. Siricidae; 1,485. |
| [61] |  |
| [62] |  |
| [63] | Submission of treatment for approval |
| [64] |  |
| [65] | All treatments proposed for inclusion in ISPM 15 should be submitted following the guidance provided in ISPM 28. |

Link to IPPC Procedure Manual Standard Setting